Project Documentation: The Spy’s Echo (Reflection)

Objective

This simulation models how a wireless signal carrying a secret message behaves when it reflects off a metallic wall. The objective is to analyze wave reflection, identify antinodes (points of maximum amplitude), and determine the optimal receiver position to successfully decode the hidden message.

Members (A2)

- Akli Merouane

- El Mokretar Akram

- Saibi Mohamed Abdelilah

- Akboudj Anis Mohammed Nassim

- Messikh Mohammed Yahia

- Namane Haithem

- Bennacer Sami Fares

1. Physical Model and Theoretical Foundations

Law of Reflection

θᵢ = θᵣ

This fundamental principle from geometrical optics ensures that the incident wave reflects symmetrically from the metallic surface, preserving energy direction.

Standing Wave Model

A(x) = 2A₀ sin(kx)

- The amplitude A(x) varies with position x

- Maximum amplitude occurs at antinodes

- Minimum amplitude occurs at nodes

Wave Number

k = 2π / λ

This relates the spatial variation of the wave to its wavelength.

Antinode Positions (Derivation)

Maximum amplitude occurs when:

sin(kx) = ±1

⇒ kx = (2n + 1)π/2

⇒ x = (2n + 1)π / (2k)

This equation determines the exact spatial positions where the signal is strongest.

2. Message Encoding

Step 1: Input Message

The user inputs a plaintext message.

Step 2: Caesar Cipher

A shift-based encryption is applied.

Example:
signal → vljqdo (shift = 3)

Step 3: Frequency Mapping (FSK Justification)

Each character is mapped to a unique frequency:

- a → 300 Hz

- b → 400 Hz

- ...

- z → 2800 Hz

Justification:
Frequency Shift Keying (FSK) ensures robustness against amplitude distortion, making it suitable for transmission in reflective environments.

3. Signal Transmission (AM-FSK Model)

Signal = sin(2π f_char t) × sin(2π f_c t)

- f_char: frequency representing the character

- f_c: carrier frequency

Justification:
Combining amplitude modulation (AM) with FSK allows both frequency encoding and efficient propagation of the signal.

4. Reflection and Wave Behavior

Perpendicular Component

k⊥ = k cos(θ)

Standing Wave Amplitude After Reflection

A(x) = 2|sin(k⊥ x)|

Justification:
Only the perpendicular component contributes to interference, which creates the standing wave pattern.

5. Antinode Detection and Receiver Placement

x = (2n + 1)π / (2k⊥)

- These positions correspond to maximum signal strength

- The optimal receiver position is the first antinode (n = 0)

Justification:
Placing the receiver at an antinode maximizes signal amplitude, improving decoding accuracy.

6. Signal Reception

A = sw_amp(x, θ)

- If A ≥ threshold → signal is strong (reliable decoding)

- If A < threshold → signal is weak (possible data loss)

7. Demodulation Process

Steps:

- Multiply the received signal by the carrier signal

- Apply a low-pass filter

Justification:
This process removes the carrier frequency and retrieves the original baseband signal.

8. Filtering

A low-pass filter eliminates high-frequency noise and preserves the useful signal components.

9. Frequency Analysis (DFT)

Using Fourier Transform:

- Extract frequency components

- Detect peaks corresponding to transmitted characters

Justification:
Each character has a unique frequency, so identifying peaks allows accurate symbol recovery.

10. Decoding

- If amplitude is sufficient → message is decoded correctly

- Otherwise → message becomes corrupted (????)

Decryption is performed using the inverse Caesar cipher.

11. Visualization Panels

The simulation interface includes:

- TX Panel: transmitted signal

- Geometry Panel: reflection visualization

- Standing Wave Panel: nodes and antinodes

- DFT Panel: frequency spectrum

- RX Panel: received signal

- Status Panel: signal strength and decoding result

12. Technical Recommendations (System Security)

To strengthen the system against physical layer attacks:

- Avoid predictable reflection surfaces: Use irregular or absorbing materials to reduce standing wave formation

- Frequency hopping (FHSS): Randomly change transmission frequencies to prevent interception

- Directional antennas: Limit signal propagation to reduce unauthorized reception

- Noise injection: Add controlled noise to confuse attackers without affecting the intended receiver

- Adaptive thresholding: Dynamically adjust detection thresholds based on environment conditions

13. Conclusion

This project demonstrates how physical wave behavior (reflection and interference) can impact communication security. By combining theoretical modeling, signal processing, and simulation, we can both exploit and mitigate vulnerabilities in wireless systems.